6/5/17

Download + Installation

To Source the freesurfer environment:

- export FREESURFER_HOME=/Applications/freesurfer
source $FREESURFER_HOME/SetUpFreeSurfer.sh

- export SUBJECTS_DIR=/Users/jessicahuang/Desktop/Subjects

Configuration

License

White matter segmentation: wm.mgz

How to use recon-all: https://surfer.nmr.mgh.harvard.edu/fswiki/recon-all

Testing correct installation: https://surfer.nmr.mgh.harvard.edu/fswiki/DownloadAndInstall#TestyourFreeSurferInstallation

- recon-all -s subjid -all (Perform a full recon-all on a pre-existing subject folder)

- recon-all -i sample-001.nii.gz -s bert -all (creates a folder called bert in SUBJECTS_DIR)

  - sample-001.nii.gz is whichever directory you’re currently in

https://surfer.nmr.mgh.harvard.edu/fswiki/FreeSurferBeginnersGuide

recon-all \
  -i  <one slice in the anatomical dicom series> \
  -s  <subject id that you make up> \
  -sd <directory to put the subject folder in> \
  -all

export SUBJECT_DIR=/Users/jessicahuang/Desktop/Subjects

recon-all -i sample-001.nii.gz -s berta -all

recon-all -i WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00001.dcm -subject S02_WAH_mprage -all

Where to locate white matter segmentations

Tutorial #1: Output Data

http://freesurfer.net/fswiki/FsTutorial/OutputData_freeview

Viewing outputs

To access the viewer

cd $SUBJECTS_DIR

freeview -v \
good_output/mri/T1.mgz \
good_output/mri/wm.mgz \
good_output/mri/brainmask.mgz \
good_output/mri/aseg.mgz:colormap=lut:opacity=0.2 \
-f good_output/surf/lh.white:edgecolor=blue \
good_output/surf/lh.pial:edgecolor=red \
good_output/surf/rh.white:edgecolor=blue \
good_output/surf/rh.pial:edgecolor=red

good_output = subject name

Once inside the viewer

%Keyboard Shortcut: Alt+c will allow you to quickly cycle through all the layers. Every time you hit it, the volume at the top of the list will move to the bottom of the list.

FOR REFERENCE: here’s a successful recon-all

Commands involved:

export SUBJECTS_DIR=Desktop/Subjects

recon-all -i sample-001.nii.gz -s berta -all

Commentary

*sample-001.nii.gz was in the working directory

*berta is a new folder that’ll be created in the SUBJECTS_DIR

Try this too:

Version 1:

recon-all -i  WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00001.dcm -s S02_WAH_mprage -sd Desktop/Subjects -all

Version 2:

recon-all -i  WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00001.dcm -sd Desktop/Subjects/SO2_WAH_mprage -all

In another terminal window, try to use the freesurfer viewer! (example 4: https://surfer.nmr.mgh.harvard.edu/fswiki/DownloadAndInstall#TestyourFreeSurferInstallation )

Running on the Monster Mac:

export SUBJECTS_DIR=/Users/caramazzalab/Desktop/Jess/Subjects

cd /Users/caramazzalab/Desktop/Jess/RAW_data/S02_WAH_mprage/

Desktop 1:

recon-all \

-i WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00001.dcm \

.

.

.

-i WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00192.dcm \

-s S02_WAH_mprage \

-all

Desktop 2:

recon-all \

-i WXY203^ChouYanrong-0003-0001-00001.dcm \

.

.

.

-i WXY203^ChouYanrong-0003-0001-00192.dcm \

-s S02_QYR_mprage \

-all

Methods of Running Recon-all:

*does work but isn’t very automatic

recon-all \

-i WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00001.dcm \

.

.

.

-i WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00192.dcm \

-s S02_WAH_mprage \

-all

**doesn’t work, only one file found

recon-all \

-i WXY207_WangAihong-1004-t1_mprage_sag_iso_mww-00001.dcm \

-s S02_WAH_mprage \

-all

**doesn’t work, there needs to be an ‘-i’ in front of all files

recon-all \

-i /Users/caramazzalab/Desktop/Jess/RAW_data/S02_WAH_mprage/* \

-s S02_WAH_mprage \

-all

**doesn’t work, you need the -i flag

recon-all \

-i /Users/caramazzalab/Desktop/Jess/RAW_data/S02_WAH_mprage/ \

-s S02_WAH_mprage \

-all

<pending new method featuring a bash script! See https://docs.google.com/a/college.harvard.edu/document/d/1WTTz6f27SL8nVFojtYCExjpmmEY7IjZg5LJD2RvHoFU/edit?usp=sharing >

Time Scale of recon-all .nifti

On my mac:

| Step | Time Stamp |
| --- | --- |
| Autorecon Processing Stages (see -autorecon# flags above): 1. Motion Correction and Conform 2. NU (Non-Uniform intensity normalization) 3. Talairach transform computation 4. Intensity Normalization 1 5. Skull Strip 6. EM Register (linear volumetric registration) 7. CA Intensity Normalization 8. CA Non-linear Volumetric Registration 9. Remove neck 10. EM Register, with skull 11. CA Label (Aseg: Volumetric Labeling) and Statistics 12. Intensity Normalization 2 (start here for control points) 13. White matter segmentation 14. Edit WM With ASeg 15. Fill (start here for wm edits) 16. Tessellation (begins per-hemisphere operations) 17. Smooth1 18. Inflate1 19. QSphere 20. Automatic Topology Fixer 21. White Surfs (start here for brain edits for pial surf) 22. Smooth2 23. Inflate2 24. Spherical Mapping 25. Spherical Registration 26. Spherical Registration, Contralater hemisphere 27. Map average curvature to subject 28. Cortical Parcellation (Labeling) 29. Cortical Parcellation Statistics 30. Pial Surfs 31. WM/GM Contrast 32. Cortical Ribbon Mask 33. Cortical Parcellation mapped to ASeg 34 Brodmann and exvio EC labels | #-------------------------------------------- #@# MotionCor Mon Jun 5 15:30:38 EDT 2017 #-------------------------------------------- #@# Talairach Mon Jun 5 15:30:47 EDT 2017 #-------------------------------------------- #@# Nu Intensity Correction Mon Jun 5 15:32:18 EDT 2017 #-------------------------------------------- #@# Intensity Normalization Mon Jun 5 15:34:16 EDT 2017 #-------------------------------------------- #@# Skull Stripping Mon Jun 5 15:36:53 EDT 2017 #------------------------------------- #@# EM Registration Mon Jun 5 15:59:16 EDT 2017 #-------------------------------------- #@# CA Normalize Mon Jun 5 16:19:54 EDT 2017 #-------------------------------------- #@# CA Reg Mon Jun 5 17:15:26 EDT 2017 #-------------------------------------- #@# SubCort Seg Mon Jun 5 22:43:31 EDT 2017 #-------------------------------------- #@# Merge ASeg Tue Jun 6 10:09:10 EDT 2017 \n cp aseg.auto.mgz aseg.presurf.mgz \n #-------------------------------------------- #@# Intensity Normalization2 Tue Jun 6 10:09:10 EDT 2017 #-------------------------------------------- #@# Mask BFS Tue Jun 6 10:13:00 EDT 2017 #-------------------------------------------- #@# WM Segmentation Tue Jun 6 10:13:02 EDT 2017 #-------------------------------------------- #@# Fill Tue Jun 6 10:15:29 EDT 2017 #-------------------------------------------- #@# Tessellate lh Tue Jun 6 10:16:15 EDT 2017 #-------------------------------------------- #@# Tessellate rh Tue Jun 6 10:16:22 EDT 2017 #-------------------------------------------- #@# Smooth1 lh Tue Jun 6 10:16:29 EDT 2017 #-------------------------------------------- #@# Smooth1 rh Tue Jun 6 10:16:39 EDT 2017 #-------------------------------------------- #@# Inflation1 lh Tue Jun 6 10:16:49 EDT 2017 #-------------------------------------------- #@# Inflation1 rh Tue Jun 6 10:17:24 EDT 2017 #-------------------------------------------- #@# QSphere lh Tue Jun 6 10:18:00 EDT 2017 #-------------------------------------------- #@# QSphere rh Tue Jun 6 10:21:36 EDT 2017 |

FINAL: RECON-ALL SUBJECTS PROCEDURE

Using script: recon-all-dicom.sh inside RAW_data

It’s a scanner, so put in the folder name and you’ll be all set

VIEWING SUBJECTS

$> cd $SUBJECTS_DIR
$> freeview -v \
    bert/mri/T1.mgz \
    bert/mri/wm.mgz \
    bert/mri/brainmask.mgz \
    bert/mri/aseg.mgz:colormap=lut:opacity=0.2 \
    -f \
    bert/surf/lh.white:edgecolor=blue \
    bert/surf/lh.pial:edgecolor=red \
    bert/surf/rh.white:edgecolor=blue \
    bert/surf/rh.pial:edgecolor=red

Guide to commands with viewing: https://surfer.nmr.mgh.harvard.edu/fswiki/FreeviewGuide/FreeviewGeneralUsage/FreeviewQuickStart

Icons: https://surfer.nmr.mgh.harvard.edu/fswiki/FreeviewGuide/FreeviewTools/VoxelEdit

Editing segmentation and parcellation: https://surfer.nmr.mgh.harvard.edu/fswiki/FreeviewGuide/FreeviewTools/VoxelEdit

GENERAL EDITING

https://surfer.nmr.mgh.harvard.edu/fswiki/Edits

EDITING PIAL SURFACES

https://surfer.nmr.mgh.harvard.edu/fswiki/FsTutorial/PialEdits_tktools

http://freesurfer.net/fswiki/FsTutorial/PialEdits_freeview

EDITING WHITE MATTER SURFACES

https://surfer.nmr.mgh.harvard.edu/fswiki/FsTutorial/WhiteMatterEdits_freeview

VIEWING 3D SURFACES

http://freesurfer.net/fswiki/FsTutorial/OutputData_freeview

Relevant files to load

Volumes

T1.mgz

brainmap.mgz

Surfaces

lh.white

lh.pial

rh.white

rh.pial

lh.inflated

rh.inflated

Good commands to color the lines automatically:

$> cd $SUBJECTS_DIR
$> freeview -v \
    bert/mri/T1.mgz \
    bert/mri/wm.mgz \
    bert/mri/brainmask.mgz \
    bert/mri/aseg.mgz:colormap=lut:opacity=0.2 \
    -f \
    bert/surf/lh.white:edgecolor=blue \
    bert/surf/lh.pial:edgecolor=red \
    bert/surf/rh.white:edgecolor=blue \
    bert/surf/rh.pial:edgecolor=red

Want to run recon-all to create just one more new file?

http://freesurfer.net/fswiki/ReconAllTableStableV5.1 here are the flags

Here’s a relevant FAQ: http://freesurfer.net/fswiki/UserContributions/FAQ#Q.Oneormoreofthefilesinasubject.27sdirectorywasaccidentallydeletedorhasbecomecorrupted.HowdoIrecreatethemissingfile.28s.29.3F

Transitioning to matlab: Q. Can I load FreeSurfer output in Matlab?

A: If you write it as .mgz format then load_mgh (or MRIread) will read it

The problem with all the errors!

https://mail.nmr.mgh.harvard.edu/pipermail//freesurfer/2015-November/042168.html

Only feed in one dicom file - if you feed in more than one, you’ll get the (identical images?) error

Which dicom file do you feed into recon-all?

Figure it out based on this message: https://mail.nmr.mgh.harvard.edu/pipermail//freesurfer/2014-March/036583.html

dcmunpack -src /Users/caramazzalab/Desktop/Jess/RAW_data/S02_WAH_mprage → shows first dicom file HM let’s see if this works by just passing in *

To view subjects use the following scripts:

An interactive script for viewing pial and wm is /Users/caramazzalab/Desktop/Jess/Subejcts/view-pial-wm.sh. An interactive script for viewing inflated hemispheres is

What code do I run after I spot an error in freesurfer?

A cheat sheet on troubleshooting: https://surfer.nmr.mgh.harvard.edu/fswiki/FsTutorial/TroubleshootingData

After pial surface: recon-all autorecon3.pial

Error #1: Too much included in gray matter

Upper brain scans:

- How to fix: erasing dura/blood vessels/optic tract

  - Edit brainmask.mgz and erase dura/blood vessels

  - Run: recon-all -s <subejct> -autorecon-pial

- Characterize the problem:

  - Dura at the top of the brain is included in the gray matter

Middle brain scans:

- How to fix: erasing dura/blood vessels/optic tract

  - Edit brainmask.mgz and erase dura/blood vessels

    - Follow instructions on editing brainmask.mgz here: https://surfer.nmr.mgh.harvard.edu/fswiki/FsTutorial/PialEdits_freeview

    - Click edit button with the R inside a brain

    - Hit brush size = 2

    - Check off recon editing

    - Make sure brainmask.mgz is highlighted on the left

    - Hold down shift and left click voxels you want to clear

    - File save volume

  - Run: recon-all -s <subject> -autorecon-pial

- Characterizing the problem:

  - Optic tract is not gray matter but is clustered as gray matter

Error #2: Not enough included in white matter

To fix lower brain scans:

- How to fix

  - Add control points to the place you know should be marked as white matter

    - Here are directions on adding control points https://surfer.nmr.mgh.harvard.edu/fswiki/FsTutorial/ControlPoints_freeview

    - File > new point set > control.dat

    - File > save point set > control.dat

  - Save control points as a file bert/tmp/control.dat

  - Run: recon-all -s <subject> -autorecon2-cp -autorecon3

- Characterizing the problem:

  - Towards the bottom slices of the brain, intensity differences between white matter and gray matter diminishes - white matter starts to look darker/ more like gray matter

  - Ex. the temporal lobe often is missing gray matter, so the move to extend the temporal lobe is to extend the white matter

  - Sometimes, white matter is missed, instead marked as gray matter, AND surrounding gray matter is missed too

Run for error 1 and 2 combined :

export FREESURFER_HOME=/Applications/freesurfer

source $FREESURFER_HOME/SetUpFreeSurfer.sh

export SUBJECTS_DIR=/Users/caramazzalab/Desktop/Jess/subjects

recon-all -autorecon-pial -autorecon2-cp -autorecon3  -s S02_WAH_mprage

export FREESURFER_HOME=/Applications/freesurfer

source $FREESURFER_HOME/SetUpFreeSurfer.sh

export SUBJECTS_DIR=/Users/caramazzalab/Desktop/Jess/subjects

recon-all -autorecon-pial -autorecon2-cp -autorecon3 -clean-cp -s S05_XYZ_mprage

export FREESURFER_HOME=/Applications/freesurfer

source $FREESURFER_HOME/SetUpFreeSurfer.sh

export SUBJECTS_DIR=/Users/caramazzalab/Desktop/Jess/subjects

recon-all -autorecon-pial -s B204_mprage

export FREESURFER_HOME=/Applications/freesurfer

source $FREESURFER_HOME/SetUpFreeSurfer.sh

export SUBJECTS_DIR=/Users/caramazzalab/Desktop/Jess/subjects

recon-all -autorecon-pial -autorecon2-cp -autorecon3  -s S06_YBH_mprage

Error #3: White matter defects - non continuity - sulci looks like WM (handles), WM looks like gray matter (holes)

- How to fix

  - Edit wm.mgz - for holes, fill voxels - for handles, erase voxels

  - Run: recon-all -s <subject> -autorecon3-pial

- Characterizing the problem

  - Hole: WM looks like gray matter

  - Handle: Some sulci is wrongly classified as WM

Next Steps: Functional Preprocessing

How do you load slice time into the pipeline?

How do you do multiband?